Internet

►Medium Access Control protocol：	Avoid collisions or re-transmit data if a collision occurs,

Offer each node a fair access to the channel. (Each device on the network gets a fair share of channel bandwidth on average.)

►Bandwidth: The amount of data that can be passed along a communication channel in a given period of time.
►Channel access methods: MAC protocols follow different approaches for sharing the channel.

Random Access: 	Stations contend with each other without any centralized coordination. (Collisions are the norm)

A specific algorithm for resolving contention/reducing collisions once they happen.

resolve collision冲突: detect a collision and do something to fix it	reduce collisions: reduce the odds for a collision to happen
	Deterministic Access: 	There is no contention – stations agree in advance (There are no collisions)

Different ways to agreeing, resulting in different MAC protocols:

Centralized: a unique entity decides on resource allocation		Distributed: nodes agree by exchanging messages

►ALOHA network: Use the direct form of transmitting user information in a single high-speed packet burst in a shared wireless channel

Channel access philosophy: let collisions happen, detect when they occur and then try again.

Any station can send data at any time

If, while transmitting, any data is received concurrently, then there is a collision – will need to try again

How to try again?

Re-send data after a random duration called the Backoff period

Avoids repeated collisions.

The way this random choice is made influences the overall performance.
	Vulnerability period: The message transmitted at time t experiences a collision if any other message overlaps partially with its transmission.

If all messages have equal length T, then the vulnerability period is of size 2T.
	Throughput: Suppose that the number of transmission attempts per frame duration T follows a Poisson distribution of mean G.

Thus, the probability of having k attempts during T is: Gke-Gk!

The probability of having no collision for the vulnerability period of 2T is given by e-2G

Thus, the throughput is the number G of attempts during T that don’t experience any collision: D=G*e-2G

提高ALOHA效率: reduce the vulnerability period duration by synchronizing

All nodes are synchronized on a given slot duration of size T.

A transmission can only start at slot begin. → vulnerability period is reduced to T.
					Throughput: D=G*e-G

The number of transmissions E: E= k=1∞kPk=eG , Pk=e-G  1-e-G k-1

►Carrier Sense Multiple Access: The node has to sense the channel to detect an ongoing transmission

If the channel is busy, then she defers transmission;			If the channel is idle, then she transmits

Whenever a node starts transmitting, it sends the complete message.

0